2025/7/3 11:01

Swagger UI

./opt/swagger-conf

Explore

# Slamware RESTful API 1.1.0     OAS3

./opt/swagger-conf.json

本⽂档适⽤于思岚科技旗下的各类底盘和服务机器⼈产品.

system 系统资源

| GET /api/core/system/v1/capabilities 获取机器⼈能⼒ |  |  |
| --- | --- | --- |
| 该接⼝⽤于判断机器⼈⽀持哪些功能， 以及是否已完成初始化。本⽂档中的部分接⼝需要依赖特定的capability才能运⾏。 |  |  |
| Parameters |  |  |
|  | Try it out |  |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema [ { "name " : "slamware.agent.core " , "version " : "4 .0 .0 " , "enabled " : true } ] |  |  |

| GET /api/core/system/v1/power/status | 获取机器⼈电源状态 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameters |  |  |  |  |  |
|  |  |  |  | Try it out |  |

https://docs.slamtec.com/#/industry/postTaskEvent                                                                                                                                                                                                   1/115

| No parameters Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK Media type |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "batteryPercentage " : 90 , "dockingStatus " : "on_dock" , "isCharging " : true , "isDCConnected " : false , "powerStage " : "running " , "sleepMode " : "awake " } |  |  |
| POST /api/core/system/v1/power/:shutdown |  | 关闭或重启机器⼈ |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |
| 通过设置关机时间和重启时间来实现机器⼈延时重启， Example Value Schema { "shutdown_time_interval " : 0 , "restart_time_interval " : 0 } Responses Code Description |  | 单位分钟，如果都为0则表示⽴即关机且不再重启。 Links |
| 200 OK Media type |  | No links |

| Code Description Links application/json. Controls Accept header. Example Value Schema true |
| --- |
| POST /api/core/system/v1/power/:hibernate 休眠机器⼈ |  |
| 休眠时激光雷达暂停⼯作 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description Links 200 No links OK |  |

| POST /api/core/system/v1/power/:wakeup | 唤醒机器⼈ |  |
| --- | --- | --- |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description 200 OK |  | Links No links |

| POST /api/core/system/v1/power/:restartmodule 重启模块 |  |  |
| --- | --- | --- |
| Parameters |  |  |
|  | Try it out |  |

| No parameters |  |  | application/json. |
| --- | --- | --- | --- |
| Request body | required |  |  |
| 根据指定的重启模式（默认软复位）执⾏重启操作。 Example Value Schema { "mode " : "RestartModeSoft " } Responses Code Description |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema true |  |  |  |
| GET /api/core/system/v1/robot/info |  | 获取设备信息 |  |
| Parameters No parameters Responses Code Description |  |  | Try it out. |
|  |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema |  |  |  |

| Code Description Links { "manufacturerId " : 255 , "manufacturerName " : "Slamtec " , "modelId " : 43792 , "modelName " : "Apollo " , "deviceID " : "D2E6D7C0F7ABF29EDFEAFEFE1C781D09 " , "hardwareVersion " : "511 " , "softwareVersion " : "3 .6 .1-rtm+20210807 " } |  |
| --- | --- |
| GET /api/core/system/v1/robot/health 获取设备健康状态信息 |  |
| Parameters No parameters | Try it out. |
| Responses Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema { "hasWarning " : false , "hasError " : true , "hasFatal " : false , "baseError " : [ { "id " : 0 , "component " : 1 , "errorCode " : 33621760 , "level " : 2 , "message " : "motor barke released " } ] } |  |

| DELETE /api/core/system/v1/robot/health/{error_code} 清除出错的状态信息 |  |
| --- | --- |
| Parameters | Try it out. |
| Name Description |  |
| error_code * integer | required 错误码 |

| Name Description (path) Example : 33621760 33621760. Responses Code Description Links 200 No links OK 400 No links Invalid Error Code |
| --- |
| GET /api/core/system/v1/laserscan | 获取当前激光观测帧 |  |  |
| 所需最低固件版本 4.2.2 Parameters |  |  | Try it out. |
| No parameters Responses Code Description |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |  |
| Controls Accept header. Example Value Schema { "pose " : { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 { |  |  |  |

| Code Description Links "distance " : 0 , "angle " : 0 , "valid " : true } ] } |  |  |  |
| --- | --- | --- | --- |
| GET /api/core/system/v1/parameter 获取系统参数 |  |  |  |
| Parameters |  | Try it out. |
| Name Description |  |  |  |
| param string (query) | * required 系统参数名: base.max_moving_speed - 最⼤线速度 base.max_angular_speed - 最⼤角速度 docking.docked_register_strategy - 充电桩注册策略， always 每次回桩都 注册， when_not_exists 桩不存在时注册 |  |  |
| Available values : base.max_moving_speed, base.max_angular_speed, docking.docked_register_strategy base.max_moving_speed. |
| Responses |  |  |  |
| Code Description Links |  |  |  |
| 200 OK Media type |  | No links No links |  |
| text/plain. |  |  |
| Controls Accept header. Example Value Schema string 400 Parameter is required |  |  |  |

| PUT /api/core/system/v1/parameter 设置系统参数 |  |  |  |
| --- | --- | --- | --- |
| Parameters |  |  |  |
|  | Try it out |  |  |

| No parameters |  |  | application/json. |
| --- | --- | --- | --- |
| Request body | required |  |  |
| 设置的系统参数仅本次运⾏有效，重启机器后恢复原值 Example Value Schema { "param " : "base.max_moving_speed " , "value " : "0 .5 " } Responses Code Description |  |  | Links |
| 200 OK Media type |  |  | No links No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema true 400 Bad Request |  |  |  |
| GET /api/core/system/v1/network/status |  | 获取⽹络状态 |  |
| 获取机器⼈当前的⽹络状态 Parameters No parameters Responses |  |  | Try it out. |

| Code Description |  |  | Links |
| --- | --- | --- | --- |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema { "networkstatus " : { "ethip1 " : "192 .168 .11 .1/24 " , "ip " : "10 .6 .128 .147 " , "mac " : "ec:0e:c4:0a:e4:3b " , "mode " : "STA " , "quality " : 100 , "ssid " : "string " } } |  |  |  |
| PUT /api/core/system/v1/network/status |  | 设置⽹络状态 |  |
| 当⽹络由安卓管理时，该接⼝会返回false Parameters |  |  | Try it out. |
| No parameters |  |  |  |
| Request body Example Value | required Schema |  | application/json. |
| { "networkmode " : 0 , "options " : {} } Responses Code Description |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |
| Controls Accept header. |  |  |  |

| Code Description Links Example Value Schema |  |
| --- | --- |
| true |  |
| 400 No links |  |
| Bad Request |  |
| GET /api/core/system/v1/network/route 获取路由信息 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description Links 200 No links |  |
| OK |  |
| Media type |  |
| application/json. |
| Controls Accept header. |  |
| Example Value Schema |  |
| { |  |
| "priority " : "wifi " |  |
| } |  |
| 500 No links |  |
| Failed to get route |  |

| PUT /api/core/system/v1/network/route 设置路由信息 |  |  |
| --- | --- | --- |
| 可设置路由优先级， 当wiﬁ和4g都可⽤时，可选择wiﬁ优先或者4g优先。 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |

| Example Value Schema { "priority " : "wifi " } Responses Code Description |  | Links |
| --- | --- | --- |
| 200 |  | No links |
| OK |  |  |
| 400 |  | No links |
| Invalid JSON data |  |  |
| 500 |  | No links |
| Failed to set route |  |  |
| GET /api/core/system/v1/network/apn | 获取cmlink apn |  |
| 所需最低固件版本 4.4.0 |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description |  | Links |
| 200 |  | No links |
| OK |  |  |
| Media type |  |  |
| application/json. |  |  |
| Controls Accept header. |  |  |
| Example Value Schema |  |  |
| 500 |  | No links |
| Failed to get cmlink apn |  |  |

| Code Description Links |  |  |  |
| --- | --- | --- | --- |
| PUT /api/core/system/v1/network/apn 设置cmlink apn |  |  |  |
| 根据地区来设置cmlink apn ，设置4g在不同地区的接⼊点，具体的apn请查阅运营商官⽹ 所需最低固件版本 4.4.0 |  |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |  |
| Request body | required | application/json. |
| Example Value Schema { "apn " : "cmhk" } |  |  |  |
| Responses |  |  |  |
| Code Description Links 200 No links OK 500 No links Failed to set cmlink apn |  |  |  |

| PUT /api/core/system/v1/cube/config 设置Cube配置 |  |
| --- | --- |
| 以⼆进制⽅式读取cube_cfg_dat⽂件作为Request Body. Cube配置⽂件请⽤RoboStudio的Cube配置⼯具导出或联系思岚技术⽀持获取. 所需最低固件版本 4.2.0 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/octet-stream. |

| Example values are not available for application/octet -stream media types. Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK |  | No links |
| POST /api/core/system/v1/light/control 设置灯光控制效果 |  |  |
| 可以设置不同通道，不同部分，不同类型的led灯颜⾊效果。 Parameters |  | Try it out. |
| No parameters |  |  |
| Request body Example Value | required Schema | application/json. |
| { "channel " : "One " , "controlPart " : "Left " , "mode " : "AlwaysBright " , "color " : { "red " : 0 , "green " : 0 , "blue " : 0 } , "brightnessEndColor " : { "red " : 0 , "green " : 0 , "blue " : 0 } , "brightMs " : 0 , "offMs " : 0 } Responses Code Description |  | Links |
| 200 OK 400 Invalid JSON data |  | No links No links |

| Code Description Links 500 No links Failed to set light control |  |  |  |
| --- | --- | --- | --- |
| POST /api/core/system/v1/aeb/control 设置AEB控制 |  |  |  |
| 设置AEB功能打开或者关闭。 |  |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |  |
| Request body | required | application/json. |
| Example Value Schema "On " |  |  |  |
| Responses |  |  |  |
| Code Description Links 200 No links OK 400 No links Invalid JSON data 500 No links Failed to set aeb control |  |  |  |

| POST /api/core/system/v1/jack/status 设置千⽄顶状态 |  |  |
| --- | --- | --- |
| 设置千⽄顶状态。 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |

| Example Value Schema "On " Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK 400 Invalid JSON data 500 Failed to set aeb control |  | No links No links No links |
| GET /api/core/system/v1/jack/status | 获取千⽄顶状态 |  |
| 获取千⽄顶状态。 Parameters No parameters Responses |  | Try it out. |

| Code Description |  | Links No links |
| --- | --- | --- |
| 200 OK Media type |  |  |
| application/json. |  |
| Controls Accept header. Example Value Schema { "actual_pos " : 0 , "alarm " : 0 , "drv_status " : 0 , "stage " : 0 } |  |  |
| GET /api/core/system/v1/rawadcimu | 获取IMU的ADC原始值 |  |
| 获取机器⼈IMU的ADC原始值 Parameters |  | Try it out. |
| No parameters Responses Code Description |  | Links |
| 200 OK Media type |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema { "acc_x " : 0 , "acc_y " : 0 , "acc_z " : 0 , "gyro_x " : 0 , "gyro_y " : 0 , "gyro_z " : 0 , "comp_x " : 0 , "comp_y " : 0 , "comp_z " : 0 , "timestamp " : 0 } |  |  |

| GET /api/core/system/v1/battery/pack |  | 获取电池包的电流和温度 |  |
| --- | --- | --- | --- |
| 获取电池包的电流和温度 |  |  |  |
| Parameters |  |  | Try it out |
| No parameters |  |  |  |
| Responses |  |  |  |
| Code Description |  |  | Links No links |
| 200 OK Media type |  |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema |  |  |  |
| { "current " : 1000 "temp_count " : 2 "temp " : [ 233 , 234 ] } | , , |  |  |

| GET /api/core/system/v1/rawimu 获取IMU原始值 |  |
| --- | --- |
| 获取机器⼈IMU原始值 |  |
| Parameters | Try it out. |
| No parameters Responses Code Description Links |  |
| 200 No links OK |  |

Code; Description; Links. Media type application/json. Controls Accept header. Example Value Schema { "acc_x " : 0 , "acc_y " : 0 , "acc_z " : 0 , "gyro_x " : 0 , "gyro_y " : 0 , "gyro_z " : 0 , "comp_x " : 0 , "comp_y " : 0 , "comp_z " : 0 , "timestamp " : 0 }.

slam 定位 、建图相关功能

| GET /api/core/slam/v1/localization/pose |  | 获取机器⼈位姿 |  |
| --- | --- | --- | --- |
| Parameters |  |  | Try it out. |
| No parameters |  |  |  |
| Responses |  |  |  |
| Code Description |  |  | Links No links |
| 200 OK Media type |  |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 |  |  |  |
|  | } |  |  |

PUT /api/core/slam/v1/localization/pose 设置机器⼈位姿

| 将机器⼈强制设置到地图中的某个位置 Parameters No parameters Request body Example Value Schema { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } Responses Code Description | Try it out. |
| --- | --- |
|  | application/json. |
|  | Links |
| 200 | No links |
| OK |  |
| 400 | No links |
| Invalid Argument |  |
| GET /api/core/slam/v1/localization/odopose 获取机器⼈⾥程计位姿 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description | Links |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. |  |

| Code Description Example Value Schema { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } |  | Links |
| --- | --- | --- |
| GET /api/core/slam/v1/localization/quality | 获取定位质量 |  |
| 定位质量范围 0 ~ 100 ，值越⼤表示定位越好 |  |  |
| Parameters |  | Try it out |
| No parameters |  |  |
| Responses |  |  |
| Code Description |  | Links |
| 200 OK Media type |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema 0 |  |  |

| GET /api/core/slam/v1/localization/:enable 是否⽀持定位 |  |
| --- | --- |
| 返回值true表示⽀持定位，false表示暂停定位即纯⾥程模式 |  |
| Parameters | Try it out. |
| No parameters |  |

| Responses Code Description Links |  |
| --- | --- |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |
| PUT /api/core/slam/v1/localization/:enable 开启/暂停定位 |  |
| 返回值true表示操作成功 Parameters No parameters Request body Example Value Schema { "enable " : true } Responses Code Description | Try it out. |
|  | application/json. |
|  | Links |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |

| Code Description Links 400 No links Bad Request |  |  |
| --- | --- | --- |
| POST /api/core/slam/v1/localization/status/:reset 重置定位状态 |  |  |
| 将定位状态重置 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |

| GET /api/core/slam/v1/mapping/:enable 是否开启建图 |  |
| --- | --- |
| 返回值true表示建图模式，false表示定位模式 |  |
| Parameters | Try it out. |
| No parameters Responses |  |

| Code Description |  | Links |
| --- | --- | --- |
| 200 OK Media type |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema true |  |  |
| PUT /api/core/slam/v1/mapping/:enable | 开启/暂停建图 |  |
| 返回值true表示操作成功 Parameters No parameters Request body Example Value Schema { "enable " : true } Responses Code Description |  | Try it out. |
|  |  | application/json. |
|  |  | Links |
| 200 OK Media type |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema true |  |  |

| Code Description Links 400 No links Bad Request |  |  |
| --- | --- | --- |
| GET /api/core/slam/v1/loopclosure/:enable 是否开启闭环检测 |  |  |
| 所需最低固件版本 4.6.0 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |

| PUT /api/core/slam/v1/loopclosure/:enable 开启/暂停闭环检测 |  |
| --- | --- |
| 返回值true表示操作成功 所需最低固件版本 4.6.0 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema |  |

| { "enable " : true } Responses Code Description | Links |
| --- | --- |
| 200 OK Media type | No links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true 400 Bad Request |  |
| GET /api/core/slam/v1/homepose 获取充电桩位置 |  |
| 获取当前的充电桩位置，如果当前地图中不存在充电桩，则返回404错误 Parameters No parameters Responses Code Description | Try it out. |
|  | Links |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema |  |

| Code Description Links |  |  |
| --- | --- | --- |
| 404 | { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } No links Home dock not found |  |
| PUT /api/core/slam/v1/homepose 设置充电桩位置 |  |  |
| 设置当前的充电桩位置， 当地图中存在多个充电桩时，需要上位机设置其中—个作为当前使⽤的桩。 |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Request body |  | application/json. |
| Example Value Schema { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } Responses Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |

| GET /api/core/slam/v1/homedocks 获取所有充电桩信息 |  |  |
| --- | --- | --- |
| 获取机器⼈的所有充电桩信息。 所需最低固件版本 4.3.2 |  |  |
| Parameters |  | Try it out |
| No parameters |  |  |
| Responses |  |  |
| Code Description 200 OK Media type |  | Links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} |  |  |
|  | } ] |  |

| PUT /api/core/slam/v1/homedocks 设置所有充电桩 |  |
| --- | --- |
| 设置机器⼈的所有充电桩信息。 所需最低固件版本 4.3.2 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema |  |

| [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } ] Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK Media type |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |
| POST /api/core/slam/v1/homedocks 添加充电桩 |  |  |
| 给机器⼈添加—个充电桩，metadata需要display_name字段，表示充电桩名称。 Parameters |  | Try it out. |
| No parameters |  |  |
| Request body Example Value | required Schema | application/json. |
| { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } |  |  |

| Responses |  |
| --- | --- |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |
| DELETE /api/core/slam/v1/homedocks 清空充电桩信息 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } |  |

POST /api/core/slam/v1/homedocks/:register 注册充电桩

根据机器⼈当前位置在地图上注册—个充电桩

| Parameters |  |  | Try it out. |
| --- | --- | --- | --- |
| No parameters |  |  |  |
| Request body Example Value | required Schema |  | application/json. |
| { "metadata " : { "display_name " : "string " } } Responses Code Description |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } |  |  |  |
| PUT /api/core/slam/v1/homedocks/{dock_id} 编辑充电桩信息 |  |  |  |
| 编辑充电桩信息，id不可修改，只允许修改pose和metadata Parameters Name Description |  |  | Try it out. |
| dock_id * required string($uuid) (path) |  | dock_id. | application/json. |
| Request body | required |  |  |

| Example Value Schema |  |  |
| --- | --- | --- |
| { |  |  |
| "pose " : { |  |  |
| "x " : 0 , |  |  |
| "y " : 0 , |  |  |
| "yaw " : 0 |  |  |
| } , |  |  |
| "metadata " : {} |  |  |
| } |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |
| DELETE /api/core/slam/v1/homedocks/{dock_id} 移除—个充电桩 |  |  |
| Parameters Name Description |  | Try it out. |
| dock_id * required string($uuid) (path) | dock_id. |
| Responses Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |  |

| Code Description Controls Accept header. Example Value Schema true | Links |
| --- | --- |
| GET /api/core/slam/v1/imu 获取IMU数据 |  |
| 获取以机器⼈坐标系表示的IMU数据 |  |
| Parameters | Try it out |
| No parameters |  |
| Responses |  |
| Code Description | Links |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema } , "availibilityBitMap " : 0 , "compass " : { "x " : 0 , "y " : 0 , "z " : 0 } , "euler_angle " : { "x " : 0 , "y " : 0 , "z " : 0 } , "gyro " : { "x " : 0 , "y " : 0 , "z " : 0 } , "quaternion " : { "w " : 0 , "x " : 0 , "y " : 0 , "z " : 0 } , "raw_acc " : { "x " : 0 , "y " : 0 |  |

| GET /api/core/slam/v1/knownarea 获取已知区域 |  |  |
| --- | --- | --- |
| 已知区域即当前地图的范围, 机器⼈的活动空间和各种⼈⼯标记元素都应当在此范围内 |  |  |
| Parameters |  |  |
|  | Try it out |  |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema { "x " : 0 , "y " : 0 , "width " : 0 , "height " : 0 } |  |  |

GET /api/core/slam/v1/maps/explore 获取栅格地图

获取激光探索的栅格地图, 可通过min_x, min_y, max_x, max_y指定获取的范围, 默认获取全部地图. 响应报⽂为⼆进制字节流，前32字节为元数据(低位字节在前) ，后续为地图数据。 位置; 数据类型; 描述. 0-3; ﬂoat; 地图起始位置的X坐标. 4-7; ﬂoat; 地图起始位置的Y坐标. 8-11; uint32; X轴⽅向栅格数量. 12-15; uint32; Y轴⽅向栅格数量. 16-19; ﬂoat; 地图分辨率，每个格⼦的边⻓ ，单位⽶. 20-31; byte[]; 预留. 32-35; uint32; 后续数据的字节数，该值应当等于X轴栅格数*Y轴栅格数. 36-End; byte[]; 地图数据，每个字节代表—个格⼦.

| Parameters |  | Try it out |
| --- | --- | --- |
| Name Description |  |  |
| min_x number | min_x. |  |
| (query) |  |  |
| min_y number | min_y. |  |
| (query) |  |  |
| max_x number | max_x. |  |
| (query) |  |  |
| max_y number | max_y. |  |
| (query) |  |  |
| Responses |  |  |
| Code Description |  | Links No links |
| 200 OK Media type |  |  |
| application/octet-stream. |  |
| Controls Accept header. Example Value Schema string |  |  |
| GET /api/core/slam/v1/maps/stcm 获取复合地图 |  |  |
| 包含所有数据的复合地图 响应报⽂为⼆进制字节流，可直接保存为stcm⽂件. |  |  |
| Parameters |  | Try it out |
| No parameters |  |  |

| Responses |  |
| --- | --- |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/octet-stream. |
| Controls Accept header. Example Value Schema string |  |
| PUT /api/core/slam/v1/maps/stcm 设置复合地图 |  |
| 将地图设置到slamware系统中, 以⼆进制⽅式读取stcm⽂件作为request body。 机器⼈位姿会被重置到原点，需要重新设置机器⼈位姿. 【注意】地图不会持久化保存，重启后即失效 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/octet-stream. |
| Example values are not available for application/octet -stream media types. |  |
| Responses |  |
| Code Description Links 200 No links OK |  |

| DELETE /api/core/slam/v1/maps 清空地图 |  |  |  |
| --- | --- | --- | --- |
| Parameters No parameters |  |  |  |
|  |  | Try it out |  |

| Responses Code Description Links 200 No links OK |  |  |
| --- | --- | --- |
| PUT /api/core/slam/v1/maps/origin 移动地图原点 |  |  |
| 移动地图原点,并更新到slamware系统中 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |
| Example Value Schema { "new_origin " : { "x " : 0 , "y " : 0 } } Responses Code Description Links |  |  |
| 200 No links OK |  |  |

artifact 地图语义元素

| GET /api/core/artifact/v1/lines/{usage} 获取虚拟线段 |  |  |
| --- | --- | --- |
| Parameters |  | Try it out. |
| Name Description |  |  |
| usage string (path) | * required | tracks 虚拟轨道 walls 虚拟墙 |

| Name Description |  |  |  |
| --- | --- | --- | --- |
| Available values : tracks, walls |  |  |  |
| tracks. |
| Responses Code Description Links |  |  |  |
| 200 OK Media type |  |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema [ { "id " : 0 , "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "metadata " : {} } ] |  |  |  |
| POST /api/core/artifact/v1/lines/{usage} 添加虚拟线段 |  |  |  |
| 添加时id为⽆效字段，可为任意值。 Parameters |  |  | Try it out. |
| Name Description |  |  |  |
| usage string (path) | * required | tracks 虚拟轨道 walls 虚拟墙 |  |
| Available values : tracks, walls |  |  |  |
| tracks. |  |
| Request body |  |  | application/json. |

| Example Value Schema [ { "id " : 0 , "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "metadata " : {} } ] Responses Code Description |  |  | Links |
| --- | --- | --- | --- |
| 200 OK Media type |  |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |  |
| PUT /api/core/artifact/v1/lines/{usage} 修改虚拟线段 |  |  |  |
| Parameters Name Description |  |  | Try it out. |
| usage string (path) | * required | tracks 虚拟轨道 walls 虚拟墙 | application/json. |
| Available values : tracks, walls |  |  |  |
| tracks. |  |
| Request body Example Value Schema |  |  |  |

| [ { "id " : 0 , "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "metadata " : {} } ] Responses Code Description Links |  |  |  |
| --- | --- | --- | --- |
| 200 OK Media type |  |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |
| DELETE /api/core/artifact/v1/lines/{usage} 清空某—类虚拟线段 |  |  |  |
| Parameters Name Description |  |  | Try it out. |
| usage string (path) | * required | tracks 虚拟轨道 walls 虚拟墙 |  |
| Available values : tracks, walls |  |  |  |
| tracks. |
| Responses |  |  |  |

| Code Description |  |  | Links |
| --- | --- | --- | --- |
| 200 OK Media type |  |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |  |
| DELETE /api/core/artifact/v1/lines/{usage}/{id} 删除虚拟线段 |  |  |  |
| Parameters |  |  | Try it out |
| Name Description |  |  |  |
| usage string (path) | * required | tracks 虚拟轨道 walls 虚拟墙 Available values : tracks, walls |  |
| id * required integer (path) |  | tracks. |  |
|  |  | id. |  |
| Responses |  |  |  |
| Code Description |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |  |

| GET /api/core/artifact/v1/rectangle-areas/{usage} 获取矩形区域 |  |  |
| --- | --- | --- |
| Parameters |  | Try it out. |
| Name Description |  |  |
| usage string (path) | * required Available values : forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area, sensor_disable_area, restricted_area |  |
| forbidden_area. |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema |  |  |
| [ { "id " : 0 , "usage " : "forbidden_area " , "area " : { "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "half_width " : 0 } , "metadata " : {} } ] |  |  |

| POST /api/core/artifact/v1/rectangle-areas/{usage} 添加矩形区域 |  |
| --- | --- |
| 不同类型的矩形区域，所需要的metadata也不同，请参考⽂档描述。 |  |
| Parameters | Try it out. |

| Name Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| usage string (path) | * required Available values : forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area, sensor_disable_area, restricted_area |  |  |  |
| Request body |  | forbidden_area. | application/json. |
| Example Value Schema { "area " : { "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "half_width " : 0 } , "metadata " : {} } Responses Code Description Links |  |  |  |  |
| 200 OK Media type |  |  | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |  |
| DELETE /api/core/artifact/v1/rectangle-areas/{usage} |  |  | 清空某—类矩形区域 |  |
| Parameters |  |  |  | Try it out. |

| Name Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| usage string (path) | * required Available values : forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area, sensor_disable_area, restricted_area |  |  |  |
| forbidden_area. Responses Code Description Links |
| 200 OK Media type |  |  | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |  |
| PUT /api/core/artifact/v1/rectangle-areas/{usage}/{id} |  |  | 编辑矩形区域 |  |
| 修改指定ID的矩形区域坐标或metadata。 |  |  |  |  |
| Parameters |  |  | Try it out. |
| Name Description |  |  |  |  |
| usage string (path) | * required | Available values : forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area, sensor_disable_area, restricted_area forbidden_area. id. |
| id * required integer (path) |  |  |  |  |
| Request body |  |  | application/json. |
| Example Value Schema |  |  |  |  |

| { "area " : { "start " : { "x " : 0 , "y " : 0 } , "end " : { "x " : 0 , "y " : 0 } , "half_width " : 0 } , "metadata " : {} } Responses Code Description Links |  |  |  |
| --- | --- | --- | --- |
| 200 OK Media type |  |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |
| DELETE /api/core/artifact/v1/rectangle-areas/{usage}/{id} 删除矩形区域 |  |  |  |
| Parameters |  |  | Try it out. |
| Name Description |  |  |  |
| usage string (path) | * required | Available values : forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area, sensor_disable_area, restricted_area forbidden_area. id. |
| id * required integer (path) |  |  |  |
| Responses |  |  |  |

| Code Description Links |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 200 OK Media type |  | No links |  |  |  |  |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |  |  |  |  |
| GET /api/core/artifact/v1/pois 获取当前地图中的所有POI |  |  |  |  |  |  |
| POI指Point of interest, 也称为星标或兴趣点，⽤于标记地图上的某个位姿， 以及若⼲与业务逻辑相关的metadata。 |  |  |  |  |  |  |
| Parameters | Try it out. |
| No parameters |  |  |  |  |  |  |
| Responses |  |  |  |  |  |  |
| Code Description Links |  |  |  |  |  |  |
| 200 OK Media type |  |  | No links |  |  |  |
| application/json. |  |  |  |  |
| Controls Accept header. Example Value Schema [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } ] |  |  |  |  |  |  |
| POST /api/core/artifact/v1/pois 添加POI |  |  |  |  |  |  |

| 调⽤⽅应当随机⽣成—个UUID作为id, metadata中的display_name⽤于界⾯显示, type⽤于区分POI类型。 在建图过程中添加POI时，建议不包含Pose ，此时会⽤机器⼈当前位置创建POI ，并且记录传感器观测信息，在闭环后会进 ⾏位姿调整。 |  |  |  |
| --- | --- | --- | --- |
| Parameters |  | Try it out. |
| No parameters |  | application/json. |
| Request body | required |  |  |
| Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } Responses Code Description Links |  |  |  |
| 200 OK |  |  | No links |
| DELETE /api/core/artifact/v1/pois |  | 清空POI |  |
| Parameters No parameters Responses Code Description |  |  | Try it out. |
|  |  |  | Links |
| 200 OK Media type |  |  | No links |
| application/json. |  |  |
| Controls Accept header. |  |  |  |

| Code Description Example Value Schema true | Links |
| --- | --- |
| POST /api/core/artifact/v1/pois/:adjust 优化POI位姿 |  |
| 如果在建图时添加POI ，则在闭环后POI会跟着调整位姿，调⽤该接⼝可以进—步减少位姿调整的误差。 【注意】仅在闭环后调⽤有效，其他时候⽆需调⽤ 。 所需最低固件版本 4.2.4 Parameters No parameters Responses Code Description 200 OK | Try it out. |
|  | Links |
|  | No links |

| GET /api/core/artifact/v1/pois/{poi_id} 根据ID查找POI |  |  |
| --- | --- | --- |
| Parameters | Try it out. |
| Name Description |  |  |
| poi_id * required string($uuid) (path) | poi_id. |
| Responses Code Description Links |  |  |
| 200 OK Media type |  | No links |

| Code Description Links application/json. Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } |
| --- |
| PUT /api/core/artifact/v1/pois/{poi_id} 修改POI |  |  |  |
| 请求报⽂中pose和metadata可以只包含其中—个，则另—个字段保持不变。 Parameters |  |  | Try it out. |
| Name Description |  |  |  |
| poi_id * required string($uuid) (path) |  | poi_id. |  |
| Request body Example Value | required Schema |  | application/json. |
| { "pose " : { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } , "metadata " : {} } Responses Code Description |  |  | Links |
| 200 OK |  |  | No links |

| Code Description Links |  |  |  |
| --- | --- | --- | --- |
| Media type |  |  |  |
| application/json. |  |
| Controls Accept header. Example Value Schema true |  |  |  |
| DELETE /api/core/artifact/v1/pois/{poi_id} 删除POI |  |  |  |
| Parameters |  | Try it out. |
| Name Description |  |  |  |
| poi_id * required string($uuid) | poi_id. |
| (path) |  |  |  |
| Responses |  |  |  |
| Code Description Links |  |  |  |
| 200 OK Media type |  | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |

| GET /api/core/artifact/v1/laser-landmarks 获取激光地标 |  |
| --- | --- |
| 激光地标指激光雷达识别到的反光板位置。 所需最低固件版本：5.1.1 |  |
| Parameters | Try it out. |

| No parameters Responses Code Description Links |  |  |
| --- | --- | --- |
| 200 OK Media type |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } ] |  |  |
| DELETE /api/core/artifact/v1/laser-landmarks | 清空激光地标 |  |
| 清空所有激光地标 所需最低固件版本：5.1.1 Parameters No parameters Responses Code Description |  | Try it out. |
|  |  | Links |
| 200 OK Media type |  | No links |
| application/json. |  |  |
| Controls Accept header. Example Value Schema |  |  |

| Code Description Links true |  |  |  |
| --- | --- | --- | --- |
| PUT /api/core/artifact/v1/laser-landmarks 设置激光地标 |  |  |  |
| 将从地图中读出的激光地标信息设置到Slamware中 所需最低固件版本：5.1.1 |  |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |  |
| Request body | required | application/json. |
| Example Value Schema [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } , "metadata " : {} } ] |  |  |  |
| Responses |  |  |  |
| Code Description Links |  |  |  |
| 200 OK Media type |  | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |

| GET /api/core/artifact/v1/laser-landmarks/:update | 获取激光地标更新状态 |  |
| --- | --- | --- |
| Slamware是否正在⾃动更新激光地标 |  |  |
| 所需最低固件版本：5.1.1 |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description |  | Links No links |
| 200 OK Media type |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema true |  |  |

| PUT /api/core/artifact/v1/laser-landmarks/:update 设置取激光地标更新状态 |  |  |
| --- | --- | --- |
| 设置是否允许Slamware⾃动更新激光地标 所需最低固件版本：5.1.1 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |
| Example Value Schema { "enable " : true } |  |  |

| Responses Code Description Links |  |  |
| --- | --- | --- |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |
| POST /api/core/artifact/v1/laser-landmarks/:remove 删除激光地标 |  |  |
| 删除部分激光地标, 请求报⽂为ID数组， ID来⾃获取激光地标接⼝返回内容的id字段。 |  |  |
| 所需最低固件版本：5.1.1 |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |
| Example Value Schema |  |  |
| [ |  |  |
| 0 |  |  |
| ] |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. |  |  |

| Code | Description | Links |  |
| --- | --- | --- | --- |
|  | Example Value Schema true |  |  |

motion 机器⼈运动控制

| GET /api/core/motion/v1/action-factories 获取所有⽀持的Action |  |  |
| --- | --- | --- |
| Parameters |  | Try it out |
| No parameters |  |  |
| Responses |  |  |
| Code Description 200 OK Media type |  | Links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema [ { "action_name " : "slamtec.agent.actions.MoveToAction " |  |  |
|  | } ] |  |

| GET /api/core/motion/v1/actions/:current 获取当前⾏为 |  |  |
| --- | --- | --- |
| Parameters No parameters Responses |  |  |
|  | Try it out |  |

| Code Description Links |  |
| --- | --- |
| 200 OK Media type | No links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "action_id " : 0 , "action_name " : "string " , "stage " : "GOING_TO_TARGET " , "state " : { "status " : 0 , "result " : 0 , "reason " : " " } } 404 Action Not Found |  |
| DELETE /api/core/motion/v1/actions/:current 终⽌当前⾏为 |  |
| Parameters No parameters | Try it out. |
| Responses Code Description Links |  |
| 200 No links OK |  |

| POST /api/core/motion/v1/actions 创建新的运动⾏为 |  |  |
| --- | --- | --- |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Request body | required | application/json. |
| action_name通过/core/motion/v1/action-factories接⼝进⾏查询, options具体内容根据action类型⽽定 Example Value Schema |  |  |

| { "action_name " : "slamtec.agent.actions.MoveToAction " , "options " : { "target " : { "x " : 0 , "y " : 0 , "z " : 0 } , "move_options " : { "mode " : 0 , "flags " : [] , "yaw " : 0 , "acceptable_precision " : 0 , "fail_retry_count " : 0 , "speed_ratio " : 0 } } } Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK Media type |  | No links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "action_id " : 0 , "action_name " : "string " , "stage " : "GOING_TO_TARGET " , "state " : { "status " : 0 , "result " : 0 , "reason " : " " } } 400 Can not create action |  |  |
| GET /api/core/motion/v1/actions/{action_id} 查询Action状态 |  |  |
| 可查询最近20次action的状态, state.status为4表示action已结束，此时通过result判断成功与否。 Parameters Name Description |  | Try it out |
| action_id * required integer | action_id. |

| Name Description |  |  |  |
| --- | --- | --- | --- |
| (path) Responses Code Description |  | Links |  |
| 200 OK Media type |  | No links No links |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema |  |  |  |
| 404 | { "action_id " : 0 , "action_name " : "string " , "stage " : "GOING_TO_TARGET " , "state " : { "status " : 0 , "result " : 0 , "reason " : " " } } Not Found |  |  |
| GET /api/core/motion/v1/path 获取剩余路径点 |  |  |  |
| 当前Action剩余的路径点 Parameters No parameters Responses Code Description |  | Try it out. |  |
|  |  | Links |  |
| 200 OK Media type |  | No links |  |
| application/json. |  |  |

| Code Description Links Controls Accept header. Example Value Schema { "path_points " : [ [ 0 , 0 ] ] } |  |
| --- | --- |
| GET /api/core/motion/v1/milestones 获取剩余⽬标点 |  |
| 当前Action剩余的⽬标点 |  |
| Parameters | Try it out. |
| No parameters Responses Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema { "path_points " : [ [ 0 , 0 ] ] } |  |

| GET /api/core/motion/v1/speed 获取运动速度 |  |
| --- | --- |
| 获取机器⼈当前运动速度 |  |
| Parameters | Try it out. |

| No parameters |  |
| --- | --- |
| Responses |  |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema |  |
| { "vx " : 0 "vy " : 0 "omega " } | , , : 0 |
| GET /api/core/motion/v1/time 获取剩余时间 |  |
| 获取机器⼈到⽬的地的剩余运动时间（估计值） |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema 0 |  |

| POST /api/core/motion/v1/:search_path |  | 搜索路径 |  |
| --- | --- | --- | --- |
| 搜索从机器⼈到⽬标点的最优路径 |  |  |  |
| Parameters |  |  | Try it out. |
| No parameters |  |  |  |
| Request body | required |  | application/json. |
| Example Value Schema { "target " : { "x " : 0 , "y " : 0 } , "timeout " : 0 } |  |  |  |
| Responses |  |  |  |
| Code Description |  |  | Links No links |
| 200 OK Media type |  |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema { "path_points " : [ [ 0 , 0 ] ] } |  |  |  |

GET /api/core/motion/v1/strategies 获取⽀持的所有运动策略

运动策略为Slamware—系列内部参数的组合，涉及到运动速度 、避障⾏为等各个⽅⾯，不同的策略可适⽤于不同的场景。 —般情况下采⽤默认策略即可。 所需最低固件版本 4.2.4

| Parameters | Try it out. |
| --- | --- |
| No parameters |  |
| Responses |  |
| Code Description | Links No links |
| 200 OK Media type |  |
| application/json. |  |
| Controls Accept header. Example Value Schema [ "default " ] |  |
| GET /api/core/motion/v1/strategies/:current 获取当前运动策略 |  |
| Parameters | Try it out |
| No parameters |  |
| Responses |  |
| Code Description | Links No links |
| 200 OK Media type |  |
| text/plain. |  |
| Controls Accept header. Example Value Schema string |  |

PUT /api/core/motion/v1/strategies/:current 设置运动策略

| Parameters |  |  |  | Try it out |  |
| --- | --- | --- | --- | --- | --- |
| No parameters |  |  |  |  |  |
| Request body | required | application/json |  |  |  |
| Example Value Schema { "strategy " : "string " } |  |  |  |  |  |
| Responses |  |  |  |  |  |
| Code Description Links |  |  |  |  |  |
| 200 OK Media type |  | No links |  |  |  |
| application/json. |
| Controls Accept header. Example Value Schema true |  |  |  |  |  |

ﬁrmware 固件升级

查询新版本固件 GET /api/core/firmware/v1/newversion

GET /api/core/firmware/v1/autoupdate/:enable 是否⽀持⾃动升级

PUT /api/core/firmware/v1/autoupdate/:enable 开启/关闭⾃动升级

|  |  | POST | /api/core/firmware/v1/autoupdate/:start 开始⾃动固件升级 |  |
| --- | --- | --- | --- | --- |
|  | POST |  | /api/core/firmware/v1/update/:start 上传固件升级 |  |
|  | GET |  | /api/core/firmware/v1/progress 获取固件升级进度 |  |

statistics 运⾏数据统计

| GET /api/core/statistics/v1/odometry | 获取运⾏⾥程 |  |
| --- | --- | --- |
| 机器⼈总的运⾏⾥程，单位⽶ |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description |  | Links No links |
| 200 OK Media type |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema 0 |  |  |

| GET /api/core/statistics/v1/runtime 获取运⾏时间 |  |
| --- | --- |
| 机器⼈总的运⾏时间，单位秒 |  |
| Parameters | Try it out. |
| No parameters Responses Code Description Links |  |
| 200 No links OK |  |

| Code | Description | Links |
| --- | --- | --- |
|  | Media type |  |
|  | application/json |  |
|  | Controls Accept header. Example Value Schema 0 |  |

sensors 传感器控制

| PUT /api/core/sensors/v1/depth/:enable 使能/禁⽤深度摄像头数据 |  |
| --- | --- |
| ⽤户设置是否使⽤深度摄像头数据 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema { "enable " : true } |  |
| Responses |  |
| Code Description Links 200 No links OK |  |
| GET /api/core/sensors/v1/masks 获取传感器禁⽤状态 |  |
| 获取禁⽤状态的传感器掩码信息。 |  |
| Parameters | Try it out. |

| No parameters Responses Code Description | Links |
| --- | --- |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema [ { "id " : 0 , "isAlways " : true } ] |  |
| PUT /api/core/sensors/v1/masks 使能/禁⽤传感器 |  |
| 设置传感器掩码。 Parameters No parameters Request body Example Value Schema [ { "id " : 0 , "isAlways " : true , "isEnabled " : true } ] Responses Code Description | Try it out. |
|  | application/json. |
|  | Links |
| 200 OK | No links |

Code Description Links

Media type

application/json.

Controls Accept header. Example Value Schema true

application 安卓应⽤程序管理(仅限ARM平台)

| GET /api/core/application/v1/apps |  | 获取所有⾃定义安装的APP |  |
| --- | --- | --- | --- |
| Parameters |  |  | Try it out |
| No parameters |  |  |  |
| Responses |  |  |  |
| Code Description |  |  | Links No links |
| 200 OK Media type |  |  |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema [ { "name " : "string " , "version " : "string " |  |  |  |
|  | } ] |  |  |

| POST /api/core/application/v1/apps 安装APP |  |
| --- | --- |
| Parameters No parameters Request body | Try it out. |
|  | application/octet-stream. |

| Example values are not available for application/octet -stream media types. |  |  |  |
| --- | --- | --- | --- |
| Responses |  |  |  |
| Code Description Links 200 No links OK 500 No links Failed to install application |  |  |  |
| DELETE /api/core/application/v1/apps/{app_name} 卸载—个APP |  |  |  |
| Parameters |  |  | Try it out. |
| Name Description |  |  |  |
| app_name string (path) | * required | app_name. |
| Responses |  |  |  |
| Code Description Links 200 No links OK |  |  |  |

platform 机器⼈通⽤底盘和平台相关的功能

| GET /api/platform/v1/timestamp 获取系统时间戳 |  |
| --- | --- |
| 获取系统启动以来的毫秒数, 返回值为字符串格式的整数。 所需最低固件版本 4.2.4 |  |
| Parameters | Try it out. |
| No parameters |  |

| Responses |  |
| --- | --- |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema "string " |  |
| GET /api/platform/v1/events 获取事件信息 |  |
| 获取机器⼈发⽣的事件，上位机可以播报语⾳或进⾏别的交互，启⽤不同的插件会扩展出不同的事件类型。 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema [ { "type " : "DEVICE_ERROR" , "timestamp " : "string " } ] |  |

multi-ﬂoor 多楼层地图管理，乘电梯等功能

| GET /api/multi-floor/status 获取地图状态信息 |  |  |
| --- | --- | --- |
| Parameters |  | Try it out |
| No parameters |  |  |
| Responses |  |  |
| Code Description 200 OK Media type |  | Links No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "is_in_mapping_mode " : true , "map_load_status " : "NOT_LOADED " , "is_managed_by_cloud " : true |  |  |
|  | } |  |

| GET /api/multi-floor/map/v1/floors 获取所有楼层信息 |  |
| --- | --- |
| Parameters No parameters | Try it out. |
| Responses Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema [ { "building " : " " , |  |

| Code Description Links "floor " : "1F " , "order " : 0 , "is_default_floor " : true } ] |  |  |
| --- | --- | --- |
| GET /api/multi-floor/map/v1/floors/:current 获取机器⼈所在楼层信息 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema { "building " : " " , "floor " : "1F " , "elevator " : " " , "map_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " } |  |  |

| PUT /api/multi-floor/map/v1/floors/:current 设置机器⼈所在楼层信息 |  |  |
| --- | --- | --- |
| 正常情况下应当由机器⼈在乘坐电梯过程中⾃主切换楼层，该接⼝仅供特殊情况下（ 如⼈⼯搬运机器⼈） 使⽤ 。 |  |  |
| Parameters |  | Try it out. |
| No parameters |  | application/json. |
| Request body | required |  |
| Example Value Schema { "building " : "string " , "floor " : "string " , "pose " : { "x " : 0 , |  |  |

| "y " : 0 , "yaw " : 0 } } Responses Code Description |  | Links |
| --- | --- | --- |
| 200 OK |  | No links |
| GET /api/multi-floor/map/v1/pois 获取POI信息 |  |  |
| 通过参数指定楼层，不带参数时获取所有楼层的POI。 Parameters Name Description |  | Try it out |
| ﬂoor string (query) building string (query) | 楼层名 ﬂoor. 建筑物名 Default value : building. | Links |
| Responses Code Description |  |  |
| 200 OK Media type |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema |  |  |

| Code Description Links |  |  |  |
| --- | --- | --- | --- |
| 400 | [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "poi_name " : "string " , "type " : "ROOM " , "floor " : "string " , "building " : "string " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } } ] No links Invalid ﬂoor or building |  |  |
| POST /api/multi-floor/map/v1/pois/:search_nearby 查找最近的POI |  |  |  |
| 查找离机器⼈最近的POI信息。其中name有三个特殊值，ON_DOCK表示在桩上， IN_ELEVATOR表示在电梯内， UNKNOWN表示没有POI ，此时没有relative_pose字段，其他的值均表示地图中添加的常规POI的名称。 |  |  |  |
| Parameters |  | Try it out. |
| No parameters |  |  |  |
| Responses |  |  |  |
| Code Description Links |  |  |  |
| 200 OK Media type |  | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "name " : "ON_DOCK" , "relative_pose " : { "x " : 0 , "y " : 0 } } |  |  |  |

POST /api/multi-floor/map/v1/pois/:dispatch 查询POI的最优遍历顺序

| 给定若⼲个POI名称，返回调整顺序后的POI名称，使得机器⼈依次遍历这些POI并回到当前位置的总路径最短。 【注】该接⼝耗时随着POI数量指数增⻓ ，请勿传⼊⼤量POI。 所需最低固件版本 4.5.0 |  |  |  |
| --- | --- | --- | --- |
| Parameters |  |  | Try it out. |
| No parameters |  |  |  |
| Request body | required | application/json. |
| Example Value Schema [ "101 " , "103 " , "102 " ] Responses Code Description Links |  |  |  |
| 200 OK Media type |  |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema [ "101 " , "102 " , "103 " ] |  |  |  |
| GET /api/multi-floor/map/v1/homedocks 获取充电桩信息 |  |  |  |
| 通过Query参数指定楼层，不带参数时获取所有楼层的充电桩 Parameters Name Description |  |  | Try it out |
| ﬂoor string | 楼层名 |  |  |

| (query) Name Description ﬂoor. building 建筑物名 string (query) Default value : building. Responses Code Description Links No links 200 OK Media type application/json. Controls Accept header. Example Value Schema [ { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "dock_name " : "string " , "floor " : "string " , "building " : "string " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } } ] |
| --- |
| GET /api/multi-floor/map/v1/homedocks/:current 获取绑定的充电桩 |  |
| 获取机器⼈当前绑定的充电桩信息，如果没绑定过或dock id⽆效，返回的result为false。 |  |
| Parameters | Try it out. |
| No parameters Responses |  |

| Code | Description | Links |
| --- | --- | --- |
| 200 | OK Media type | No links |
|  | application/json. |  |
|  | Controls Accept header. Example Value Schema { "result " : true , "msg " : "string " , "data " : { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "dock_name " : "string " , "floor " : "string " , "building " : "string " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } } } |  |
| PUT | /api/multi-floor/map/v1/homedocks/:current 绑定充电桩 |  |
| 【注意】如果绑定的充电桩不在启动楼层，则需要先将机器⼈推到充电桩上， 然后调⽤本接⼝ ，此时会同步修改启动楼层并 重置地图。 |  |  |
| Parameters No parameters Request body |  | Try it out. |
|  |  | application/json. |
| Example Value Schema { "dock_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " } Responses |  |  |

| Code Description | Links |
| --- | --- |
| 200 | No links |
| OK |  |
| POST /api/multi-floor/map/v1/homedocks/:search_nearby 查找离机器⼈最近的充电桩 |  |
| 调⽤该接⼝前请确保机器⼈定位准确。 |  |
| Parameters | Try it out |
| No parameters |  |
| Responses |  |
| Code Description | Links No links |
| 200 OK Media type |  |
| application/json. |  |
| Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "dock_name " : "string " , "floor " : "string " , "building " : "string " , "pose " : { "x " : 0 , "y " : 0 , "yaw " : 0 } } |  |
| POST /api/multi-floor/map/v1/stcm 上传地图到机器⼈ |  |
| 上传的地图会持久化保存在⽂件系统中, 但不会加载到Slamware中。 【注意】 当机器⼈由云端管理时，从云端下载的地图会覆盖本地地图。 |  |
| Parameters | Try it out |
| No parameters |  |

| Request body | application/octet-stream. |
| --- | --- |
| Example values are not available for application/octet -stream media types. |  |
| Responses |  |
| Code Description 200 OK | Links |
|  | No links |
| DELETE /api/multi-floor/map/v1/stcm 删除保存的地图 |  |
| 不会清空内存中的当前地图，⽽是删除⽂件系统中缓存的地图 |  |
| Parameters | Try it out. |
| No parameters |  |
| Responses |  |
| Code Description 204 OK | Links No links |

| POST /api/multi-floor/map/v1/stcm/:save 持久化保存当前地图 |  |
| --- | --- |
| 从Slamware中读取地图并保存到⽂件。 【注意】 多楼层环境中禁⽌该操作，否则会丢失其他楼层的地图。 |  |
| Parameters | Try it out. |
| No parameters Responses |  |

| Code Description Links 200 No links OK |  |  |
| --- | --- | --- |
| POST /api/multi-floor/map/v1/stcm/:reload 重新加载地图 |  |  |
| 重新加载地图，优先尝试从云端下载，下载失败或机器⼈不受云端管理时从本地⽂件读取。 pose为可选字段，pose为空时设置机器⼈位姿到充电桩前。 【注意】 系统启动时会⾃动加载地图，该接⼝—般在部署阶段地图有变更时才需要调⽤ 。 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Request body | application/json. |
| Example Value Schema { "pose " : { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } } |  |  |
| Responses |  |  |
| Code Description Links 200 No links OK |  |  |

| POST /api/multi-floor/map/v1/stcm/:sync 同步地图 |  |
| --- | --- |
| 保存当前地图到⽂件，并重新加载，相当于save和reload 2个接⼝的组合。 【注意】 多楼层环境中禁⽌该操作，否则会丢失其他楼层的地图。 所需最低固件版本 4.2.4 |  |
| Parameters | Try it out. |

| No parameters |  |
| --- | --- |
| Responses |  |
| Code Description Links 200 No links OK |  |
| POST /api/multi-floor/map/v1/scene/unbind 解绑云端场景 |  |
| 将机器⼈与云端场景解除绑定，并删除本地地图，在机器⼈需要换场景部署时调⽤ 。 所需最低固件版本 6.2.0 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema { "keep_local_map " : true } |  |
| Responses |  |
| Code Description Links 200 No links OK |  |

| POST /api/multi-floor/map/v1/search_path_points 通过轨道搜索路径点 |  |
| --- | --- |
| 在轨道构成的图中，搜索起点到终点的可⾏路径。 |  |
| Parameters | Try it out. |

| No parameters |  |  |  |
| --- | --- | --- | --- |
| Request body | required |  | application/json. |
| Example Value Schema |  |  |  |
| { "building " : "string " , "floor " : "string " , "start_point " : { "x " : 0 , "y " : 0 } , "end_point " : { "x " : 0 , "y " : 0 } , "with_direction " : true } |  |  |  |
| Responses |  |  |  |
| Code Description |  | Links |  |
| 200 OK Media type |  | No links |  |
| application/json. |  |  |
| Controls Accept header. Example Value Schema { "path_points " : [ [ 0 , 0 ] ] } |  |  |  |
| PUT /api/multi-floor/localization/v1/pose 设置机器⼈位姿 |  |  |  |
| 将机器⼈位姿设置到指定的POI上， —般⽤于发⽣异常后的恢复操作。 所需最低固件版本 4.5.3 |  |  |  |
| Parameters | Try it out. |
| No parameters |  |  |  |
| Request body | application/json. |

| Example Value Schema |  |
| --- | --- |
| { |  |
| "poi_name " : "string " |  |
| } |  |
| Responses |  |
| Code Description Links |  |
| 200 OK Media type | No links |
| application/json. |
| Controls Accept header. Example Value Schema true |  |
| PUT /api/multi-floor/localization/v1/homedock 根据充电桩重置机器⼈定位 |  |
| 将机器⼈位姿设置到指定的充电桩前，—般⽤于发⽣异常后的恢复操作。 所需最低固件版本 6.2.0 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema { "dock_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " } Responses |  |

| Code Description Links 200 No links OK |  |  |
| --- | --- | --- |
| GET /api/multi-floor/map/v1/elevators 获取电梯区域 |  |  |
| 获取电梯区域内的元素，包括电梯ID以及等待点。 |  |  |
| Parameters | Try it out. |
| No parameters |  |  |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type | No links |  |
| application/json. |
| Controls Accept header. Example Value Schema { "door_type " : "front_door " , "elevator_id " : "string " , "front_scheduling_poses " : [ { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } ] , "rear_scheduling_poses " : [ { "x " : 0 , "y " : 0 , "z " : 0 , "yaw " : 0 , "pitch " : 0 , "roll " : 0 } ] } ] |  |  |

| GET | /api/multi-floor/map/v1/elevators/{elevator_id} 获取某个电梯的信息 |  |
| --- | --- | --- |

| Parameters |  |  |  | Try it out. |
| --- | --- | --- | --- | --- |
| Name Description |  |  |  |  |  |
| elevator_id * string (path) |  | required elevator_id. |  |  |
| Responses Code Description 200 |  |  |  | Links No links |  |
| OK |  |  |  |  |  |
| Media type |  |  |  |  |  |
| application/json. |  |  |
| Controls Accept header. |  |  |  |  |  |
| Example Value Schema |  |  |  |  |  |
| { |  |  |  |  |  |
| "door_type " : "front_door " , "elevator_id " : "string " , |  |  |  |  |  |
| "front_scheduling_poses " : [ { |  |  |  |  |  |
| "x " : 0 , "y " : 0 , "z " : 0 , |  |  |  |  |  |
| "yaw " : 0 , |  |  |  |  |  |
| "pitch " : 0 , "roll " : 0 |  |  |  |  |  |
| } |  |  |  |  |  |
| ] , |  |  |  |  |  |
| "rear_scheduling_poses " : [ { |  |  |  |  |  |
| "x " : 0 , "y " : 0 , "z " : 0 , |  |  |  |  |  |
| "yaw " : 0 , |  |  |  |  |  |
| "pitch " : 0 , "roll " : 0 |  |  |  |  |  |
| } |  |  |  |  |  |
| ] |  |  |  |  |  |
| } |  |  |  |  |  |
| 400 |  |  |  | No links |  |
| Unknown Elevator ID |  |  |  |  |  |
| GET | /api/multi-floor/map/v1/elevators/{elevator_id} /pose_relation |  | 获取机器⼈与电梯的位置关系 |  |  |
| Parameters |  | Try it out. |

| Name Description |  |  |
| --- | --- | --- |
| elevator_id * string (path) | required elevator_id. |
| Responses |  |  |
| Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |
| Controls Accept header. Example Value Schema "in_elevator " 400 No links Unknown Elevator ID |  |  |

industry ⼯业搬运服务

| POST /api/industry/v1/tasks/templates 创建任务模板 |  |
| --- | --- |
| 创建—个呼叫器任务模板 |  |
| Parameters | Try it out. |
| No parameters |  |
| Request body | application/json. |
| Example Value Schema { "key " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "name " : "string " , "action_list " : [ { "display_name " : "string " , "action " : "string " , "wait_time " : 0 |  |

| } ] } Responses Code Description | Links |
| --- | --- |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "task_template_key " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "task_template_type " : 0 , "name " : "string " , "scene_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "device_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "action_list " : [ { "display_name " : "string " , "action " : "string " , "wait_time " : 0 } ] } |  |
| GET /api/industry/v1/tasks/templates 获取任务模板 |  |
| 获取当前设备所属场景下的所有任务模板 Parameters No parameters Responses Code Description | Try it out. |
|  | Links |
| 200 OK Media type | No links |
| application/json. |  |
| Controls Accept header. |  |

| Code Description Links Example Value Schema [ { "task_template_key " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "task_template_type " : 0 , "name " : "string " , "scene_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "device_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "action_list " : [ { "display_name " : "string " , "action " : "string " , "wait_time " : 0 } ] } ] |  |  |  |
| --- | --- | --- | --- |
| DELETE /api/industry/v1/tasks/templates/{key_id} 删除任务模板 |  |  |  |
| 删除⼀个任务模板 |  |  |  |
| Parameters |  | Try it out. |
| Name Description |  |  |  |
| key_id * required string($uuid) (path) | 3fa85f64-5717-4562-b3fc-2c963f66afa6. |
| Responses |  |  |  |
| Code Description Links |  |  |  |
| 200 No links OK |  |  |  |
| Media type |  |  |  |
| text/plain. |  |  |
| Controls Accept header. Example Value Schema { "result " : true , "msg " : "string " , "data " : {} } |  |  |  |

| GET /api/industry/v1/tasks 查询任务信息 |  |  |
| --- | --- | --- |
| 默认返回ready和running状态的所有类型的任务，status为all时表示查询最近的所有任务，包括已成功完成和失败的任务。 |  |  |
| Parameters |  | Try it out. |
| Name Description |  |  |
| type string (query) status string (query) | Available values : carry_calling_by_template, carry_calling, industry --. Available values : ready, running, succeeded, failed, all |
|  | --. |
| Responses Code Description Links |  |  |
| 200 OK Media type |  | No links |
| application/json. |  |
| Controls Accept header. Example Value Schema { "id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "task" : { "target " : "string " , "type " : "CARRY_CALLING " , "order_id " : "string " , "template_key " : "string " , "start_time " : "string " , "task_targets " : [ { "target_name " : "string " , "action " : "string " , "wait_time " : 0 } ] , "message " : {} } , "status " : "READY " , "result " : { "stage " : "GOING_TO_ELEVATOR" , "reason " : " " , "timestamp " : "string " } } ] |  |  |

| POST /api/industry/v1/tasks/events 推送任务事件 |  |  |  |
| --- | --- | --- | --- |
| 上位机执⾏呼叫器任务时，通过该接⼝推送任务事件， 同时更新任务状态。 |  |  |  |
| Parameters |  |  |  |
|  |  | Try it out |  |
| No parameters |  |  |  |
| Request body | application/json |  |  |
| Example Value Schema { "task_id " : "3fa85f64-5717-4562-b3fc-2c963f66afa6 " , "status " : "RUNNING " } |  |  |  |
| Responses |  |  |  |
| Code Description Links 200 No links OK |  |  |  |

delivery 配送服务(仅限整机，通⽤底盘⽆法⽀持)

GET /api/delivery/v1/admin/password 获取操作密码

| PUT | /api/delivery/v1/admin/password 设置操作密码 |
| --- | --- |

| GET | /api/delivery/v1/admin/mode 获取机器⼈⼯作模式 |
| --- | --- |

| PUT | /api/delivery/v1/admin/mode 设置机器⼈⼯作模式 |
| --- | --- |

| GET | /api/delivery/v1/admin/language 获取机器⼈语⾔ |
| --- | --- |

| PUT | /api/delivery/v1/admin/language 设置机器⼈语⾔ |
| --- | --- |

GET /api/delivery/v1/admin/working_time 获取机器⼈⼯作时间

PUT /api/delivery/v1/admin/working_time 设置机器⼈⼯作时间

| GET | /api/delivery/v1/admin/move_options 获取运动选项 |
| --- | --- |

| PUT | /api/delivery/v1/admin/move_options 设置运动选项 |
| --- | --- |

GET /api/delivery/v1/admin/line_speed 获取配送速度和返航速度

PUT /api/delivery/v1/admin/line_speed 设置配送速度和返航速度

| GET | /api/delivery/v1/configurations 获取机器配置信息 |
| --- | --- |

| GET | /api/delivery/v1/settings 获取配送相关的设置信息 |
| --- | --- |

| PUT | /api/delivery/v1/settings/timeout 设置任务的超时时间 |
| --- | --- |

| GET | /api/delivery/v1/voice_resources 获取语⾳包信息 |
| --- | --- |

| GET | /api/delivery/v1/cargos 获取所有Cargo信息 |
| --- | --- |

GET /api/delivery/v1/cargos/{cargo_id}/boxes 获取某个Cargo所有Box信息

GET /api/delivery/v1/cargos/{cargo_id}/boxes/{box_id} 获取Box信息

PUT /api/delivery/v1/cargos/{cargo_id}/boxes/{box_id}/{op} 操作Box

| GET | /api/delivery/v1/cargos/{cargo_id}/boxes/{box_id} /operation_result | 查询Box操作结果 |
| --- | --- | --- |

| GET | /api/delivery/v1/cargos/assigned 获取被占⽤的外卖舱 |
| --- | --- |

| GET | /api/delivery/v1/tasks 查询任务信息 |
| --- | --- |

| POST | /api/delivery/v1/tasks 创建任务 |
| --- | --- |

| DELETE | /api/delivery/v1/tasks 取消所有任务 |
| --- | --- |

| POST | /api/delivery/v1/tasks/:batch 批量创建任务 |
| --- | --- |

DELETE /api/delivery/v1/tasks/{task_id} 根据Task ID取消任务

DELETE /api/delivery/v1/tasks/orders/{order_id} 根据订单ID取消任务

| GET | /api/delivery/v1/stage 获取当前任务状态 |
| --- | --- |

PUT /api/delivery/v1/tasks/:task_execution 暂停/继续执⾏任务

| PUT | /api/delivery/v1/tasks/:task_finish 结束所有任务 |
| --- | --- |

| PUT | /api/delivery/v1/tasks/:start_pickup 开始取物 |
| --- | --- |

| PUT | /api/delivery/v1/tasks/:end_pickup 结束取物 |
| --- | --- |

| PUT | /api/delivery/v1/tasks/:end_operation 完成操作 |
| --- | --- |

Schemas

Capability { name string slamware agent core 定位导航等底盘核⼼通⽤功能 · slamware agent platform ⽇志采集等平台通⽤功能 . slamware agent multi floor 多楼层地图管理和跨楼层移动功能 · slamware agent delivery 配送服务功能 · slamware.agent.mercury2 ⽀持云端调度的智能酒店配送服务功能 Enum: Array [ 5 ] version string example: 4.0.0 enabled boolean 插件初始化失败，或者刚启动时该值为false，上位机应当继续等待⼀段时间，直到该值变成true或 者超时。 }

| PowerStatus { batteryPercentage dockingStatus isCharging isDCConnected powerStage sleepMode } DeviceInfo { manufacturerId manufacturerName modelId modelName deviceID hardwareVersion softwareVersion } | integer example: 90 电池电量百分⽐ ，0 ~ 100 string 对桩状态 Enum: Array [ 2 ] boolean 是否正在充电 boolean example: false 外部电源是否连接 string example: running 电源状态 Enum: Array [ 5 ] string 休眠状态 Enum: Array [ 3 ] integer example: 255 制造商ID string example: Slamtec 制造商名称 integer example: 43792 设备型号ID string example: Apollo 设备型号名称 string($uuid) example: D2E6D7C0F7ABF29EDFEAFEFE1C781D09 设备序列号 string example: 511 硬件版本号 string example: 3.6.1-rtm+20210807 软件版本号 |
| --- | --- |

| BaseError id component | { |  | integer integer example: 1 0 User, 1 System, 2 Power, 3 Motion, 4 Sensor, 255 Unknown |
| --- | --- | --- | --- |
| errorCode level |  |  | Enum: Array [ 6 ] integer example: 33621760 integer example: 2 0 Healthy, 1 Warn, 2 Error, 4 Fatal, 255 Unknown |
| message |  |  | Enum: Array [ 5 ] string example: motor barke released |
| } |  |  |  |
| BaseHealthInfo hasWarning |  | { | boolean example: false 是否有告警信息 |
| hasError |  |  | boolean example: true 是否有错误信息 |
| hasFatal |  |  | boolean example: false |
| baseError } |  |  | 是否有致命错误 [...] |
| NetworkStatus ethip1 |  | { | string example: 192.168.11.1/24 |
| ip |  |  | 以太⽹地址 string example: 10.6.128.147 IP地址 |
| mac |  |  | string example: ec:0e:c4:0a:e4:3b |
| mode |  |  | MAC地址 string Enum: |
| quality |  |  | Array [ 2 ] integer example: 100 |
| ssid } |  |  | ⽹络质量 string |

ApModeOptions

description:

{

ssid和password均为可选项，默认从配置⽂件设置热点名称

ssid                 string

example: Slamware-58660D

password             string

}

StationModeOptions

ssid*

password*

}

{

string

example: Slamtec-Test

string

| RouteStatus priority | { | string example: wifi 路由优先级选择 : wifi - WIFI 优先 · 4g - 4G 优先 |
| --- | --- | --- |

Enum:

Array [ 2 ]

}

ApnStatus      {

apn                  string

example: cmhk

根据地区选择对应apn: 例如⾹港地区：cmhk

}

{

ActionState

status

result

reason }

ActionGravestone

action_id

state }

integer

0:NewBorn, 1:Working, 3:Paused, 4:Done Enum:

Array [ 4 ]

integer

0:Success, -1: Failed, -2: Aborted Enum:

Array [ 3 ]

string

default:

{

integer

{...}

ActionState

ActionInfo      { action_id

action_name stage

state

}

Pose3D      {

description:

x

y

z

yaw

pitch

roll }

Pose2D      {

description:

x

y

yaw

}

{

Location

x

y

z

}

{

Point

x

y }

{

Quaternion

w

x

y

z

}

[

PathPoints

integer

string

string

example: GOING_TO_TARGET

{...}

ActionState

三维空间的位姿信息

number

number

number

number

number

number

⼆维平⾯的位姿信息

number

number

number

number

number

number

number

number

number

number

number

number

x、y坐标构成的路径点，每个元素为包含两个浮点数的数组，分别为x、y坐标值 PathPoints      [...]]

LaserScan      { description:

pose为观测到该帧激光时的机器⼈位姿，每个激光点的angle表示激光与机器⼈正前⽅的夹角。

{...}

Pose3D

[...]

pose

laser_points

}

{

EmptyMetadata

description:

metadata是key和value都是字符串的map

}

BezierCurveMetadata

description:

control_point1*

control_point2*

{

描述贝塞尔曲线的metadata，control_point1和control_point2是两个控制点的坐标，再加上 Line的起点和终点可以确定—个三阶贝塞尔曲线

string

example: {'x':0.1,'y':0.1}

string

example: {'x':0.1,'y':0.1}

}

Line      {

id                   integer

如果是添加线段，id会被忽略，如果是编辑线段，则会修改对应id的线段

{...}

{...}

start

end

metadata

Point Point

{...}

}

Rectangle

x

y

width

height

{

number

number

number

number

}

RectangleAreaUsage   string

forbidden area 禁⾏区域，禁⽌机器⼈进⼊该区域 elevator area  电梯区域

dangerous area 危险区域 , 机器⼈进⼊该区域后⾃动减速

coverage area  覆盖规划区域，⽤于清扫和消毒等

maintenance area 运维区域，⽤于将建图范围限制在该区域内

sensor disable area 传感器禁⽤区域，机器⼈进⼊该区域后忽略指定的传感器数据 restricted_area 限⾏区域，⽤于多机调度，可限制同时进⼊的机器⼈数量。

Enum:

[ forbidden_area, elevator_area, dangerous_area, coverage_area, maintenance_area,

sensor_disable_area, restricted_area ]

ElevatorAreaMetadata

elevator_id*

elevator_sill_width*

{

string($uuid)

梯控设备序列号

string

example: 0.4

⻔槛宽度，从电梯⻔内侧到⻔洞外侧的距离

elevator_scheduling_point_dist* string

example: 1

电梯调度点离电梯⻔的距离

elevator_door_type             string

电梯开⻔⽅向 , 0 正⾯开⻔ ，1 背⾯开⻔ ，2 双向开⻔ Enum:

Array [ 3 ]

}

ForbiddenAreaMetadata      {

escape_distance*     string

example: 0.4

可逃脱区域⼤⼩ ，从禁区边界往⾥⾯计算，单位⽶ .

}

DangerousAreaMetadata      {

dangerous_area_type* string

危险区域类型 , 0 斜坡，1 窄⾛廊

Enum:

Array [ 2 ]

max_line_speed       string

example: 0.5

机器⼈在该区域内的最⼤线速度 , 单位⽶/秒

}

SensorDisableAreaMetadata      { sensor_type          string

example: [0，3]

传感器类型，0 碰撞，1 跌落，2 超声，3 深度

}

{

CoverageAreaMetadata

}

RestrictedAreaMetadata      {

restricted_scheduling_points  string

example: [{"x":"0.1","y":"0.1"}]

限⾏区域调度点

restricted_robots_number_limit string

example: 1

允许同时进⼊的机器⼈数量

}

RectangleAreaMetadata      {

description:

oneOf ->

}

RectangleArea

id

usage

area

metadata

}

矩形区域的metadata，键值对数据，所有数据都应当序列化成字符串格式。

| EmptyMetadata {...} ElevatorAreaMetadata ForbiddenAreaMetadata DangerousAreaMetadata CoverageAreaMetadata | {...} {...} {...} {...} |
| --- | --- |

SensorDisableAreaMetadata      {...} RestrictedAreaMetadata      {...}

{

integer

RectangleAreaUsage string

forbidden area 禁⾏区域，禁⽌机器⼈进⼊该区域 elevator area  电梯区域

dangerous area 危险区域 , 机器⼈进⼊该区域后⾃动减速

coverage area  覆盖规划区域，⽤于清扫和消毒等

maintenance area 运维区域，⽤于将建图范围限制在该区域内

sensor_disable_area 传感器禁⽤区域，机器⼈进⼊该区域后忽略指定的传感器数 据

restricted_area 限⾏区域，⽤于多机调度，可限制同时进⼊的机器⼈数量。

Enum:

Array [ 7 ]

{...}

{...}

RectangleAreaMetadata

| ImuData { description: acc availibilityBitMap compass euler_angle gyro quaternion raw_acc raw_compass raw_gyro timestamp } |  | 通过按位或运算组合⽽成的availibilityBitMap表明了哪些数据是有效的 Location {...} integer 1 以四元数表示的位姿 2 校准后的加速度计 4 校准后的陀螺仪 8 校准后的罗盘 16 加速度计原始值 32 陀螺仪原始值 64 罗盘原始值 128 6⾃由度的位姿信息 256 9⾃由度的位姿信息 512 以欧拉⻆表示的位姿 |  |
| --- | --- | --- | --- |
|  |  | Location Location Location | {...} {...} {...} |
|  |  | Quaternion {...} |  |
|  |  | Location Location Location integer | {...} {...} {...} |
|  |  | 底盘启动以来经过的毫秒数 string default: Slamtec string default: Hermes string default: 4.2.2-rtm+20211011 string($uuid) string default: string example: 1F integer 如果需要在UI中列出楼层信息，应当按此字段排序 boolean 是否为默认楼层，不指定启动楼层时，机器⼈会设置到默认楼层。所有电梯都需要停靠默认楼层。 |  |
| FirmwareInfo manufacturer model firmware firmware_id } | { |  |  |
| FloorInfo { building floor order is_default_floor } |  |  |  |

# CurrentFloorInfo

building floor

elevator

map_id

}

MultiFloorTarget

building

floor

pose }

{

string

default:

string

example: 1F

string

default:

如果该字段⾮空则表示机器⼈还在电梯内。

string($uuid)

当前所在楼层的地图ID

{

string

default:

string

example: 1F

{...}

Pose2D

PoiType   string

Enum:

[ ROOM, REFILL, RECEPTION, TABLE, PARKING, RECYCLE, DISINFECT ]

SpecialPOIName   string

ON DOCK 在桩上

IN ELEVATOR 在电梯内

UNKNOWN 未知，没有POI或软件异常

Enum:

[ ON_DOCK, IN_ELEVATOR, UNKNOWN ]

{

NearbyPoiInfo

description:

name字段有三个特殊值，除此之外都表示地图中添加的常规POI名称。 relative_pose表示POI相对 机器⼈的位姿，机器⼈前⽅为X轴正⽅向，左侧为Y轴正⽅向。

id

name*

relative_pose

string($uuid)

{...}

Point      {...}

}

MultiFloorPoiInfo

id

poi_name type

{

string($uuid)

string

PoiType string

Enum:

string

Array [ 7 ]

floor

string

Pose2D

building

{...}

pose

}

| MultiFloorDockInfo id dock_name floor building pose } |  | { string($uuid) string string string Pose2D {...} string($uuid) Pose2D {...} {...} integer string Enum: Array [ 5 ] string Enum: Array [ 2 ] string Enum: Array [ 3 ] string Enum: Array [ 3 ] [...] string($uuid) integer string Enum: Array [ 3 ] integer string Enum: Array [ 2 ] [...] [...] string($uuid) [...] |
| --- | --- | --- |
| PoseEntry id* pose | { |  |
| metadata } Box { id door_status lock_status stock_status status errors } Cargo { id pos orientation layer type errors boxes } |  |  |
| CargoEntry cargo_id boxes } | { |  |

AssignedCargoEntry

target

order_id

cargo_id

boxes

}

{

FailedTask

target

cargos

}

DeliveryTaskRequest

target

type

req_id

order_id

no_pickup_wait cargos

failed_tasks

station_id

station_cargos

message

{

string

string

string($uuid)

[...]

string

[...]

{

string

string

Enum:

Array [ 9 ]

string  string  boolean

[...]

[...]

string

[...]

{...}

}

DeliveryPickupResultEntry      { description:

包含cargo信息表示舱体的取物结果，包含task_point_id表示任务点的操作结果，⼆者只存其—

cargo_id             string($uuid)

box_id               string($integer)

example: 0

result               string

Enum:

Array [ 2 ]

reason               string

default:

task_point_id        string

default:

}

string

DeliveryTaskEventStage

任务执⾏阶段

Enum:

Array [ 13 ]

DeliveryTaskResult

stage

reason

pickup_result

timestamp

}

DeliveryTaskStatus

Enum:

Array [ 6 ]

{

DeliveryTaskEventStage string

任务执⾏阶段

Enum:

Array [ 13 ]

string

default:

[...]

string($datetime)

string

DeliveryTask

id

task

status

{

string($uuid)

DeliveryTaskRequest      {...} DeliveryTaskStatus string

Enum:

Array [ 6 ]

result

DeliveryTaskResult      {...}

}

DeviceError

component

error_code

error_level

message

{

integer

integer

integer

string

}

TaskExecutionInfo      {

enable_task_execution boolean

}

GeneralEventType   string

DEVICE ERROR 发⽣了Error或Fatal级别的设备健康状态报警

PATH OCCUPIED ⾏进路径被阻挡

ROBOT BLOCKED 在⼀个地⽅被⻓时间连续阻挡（默认3分钟）

RESET MAP TO DOCK 机器⼈被推回桩并重置地图成功

START CHARGING 开始充电

STOP CHARGING 停⽌充电

ON DOCK 机器⼈上桩

OFF DOCK 机器⼈下桩

UPGRADE 正在升级固件

POWER OFF 正在关机

PASS THE NARROW CORRIDOR  通过窄⾛廊

MAP LOOP CLOSURE  完成⼀次建图闭环

SET MAP DONE 完成设置地图操作

MOVE TO LANDING POINT_FAILED  前往充电桩失败

SEARCH DOCK FAILED 找桩失败

CHARGING BASE FAILED 充电失败

SYNC MAP FROM CLOUD 从云端同步地图

DOCK ID NOT FOUND 在地图中找不到绑定的桩

BRAKE RELEASED 刹⻋释放按钮被按下

BUMPER TRIGGERED 碰撞传感器触发

CURRENT POSE OCCUPIED  当前机器⼈位姿被占据

CLIFF_DETECTED 检测到悬崖

Enum:

[ DEVICE_ERROR, PATH_OCCUPIED, ROBOT_BLOCKED, RESET_MAP_TO_DOCK, START_CHARGING, STOP_CHARGING, ON_DOCK, OFF_DOCK, UPGRADE, POWER_OFF, PASS_THE_NARROW_CORRIDOR, MAP_LOOP_CLOSURE, SET_MAP_DONE,

MOVE_TO_LANDING_POINT_FAILED, SEARCH_DOCK_FAILED, CHARGING_BASE_FAILED, SYNC_MAP_FROM_CLOUD, DOCK_ID_NOT_FOUND, BRAKE_RELEASED, BUMPER_TRIGGERED, CURRENT_POSE_OCCUPIED, CLIFF_DETECTED ]

ElevatorEventType   string

WAIT ELEVATOR 机器⼈到达电梯等待点

ENTER ELEVATOR 即将开始进电梯

ENTER ELEVATOR OCCUPIED  进电梯过程中被阻挡

ENTER ELEVATOR FAILED 进电梯失败

TURNING ROUND IN_ELEVATOR  在电梯内即将转身

LEAVE ELEVATOR 即将开始出电梯

LEAVE ELEVATOR OCCUPIED  出电梯过程中被阻挡

LEAVE ELEVATOR_FAILED  出电梯失败

IN ELEVATOR 在电梯内

OUT OF ELEVATOR 在电梯外

TAKE ELEVATOR OCCUPIED  进出电梯被挡

SEARCH ELEVATOR PATH FAILED  搜索电梯路径失败

ENTER_ELEVATOR_PATH_FOUND 进电梯搜路成功

Enum:

[ WAIT_ELEVATOR, ENTER_ELEVATOR, ENTER_ELEVATOR_OCCUPIED, ENTER_ELEVATOR_FAILED,

TURNING_ROUND_IN_ELEVATOR, LEAVE_ELEVATOR, LEAVE_ELEVATOR_OCCUPIED, LEAVE_ELEVATOR_FAILED, IN_ELEVATOR, OUT_OF_ELEVATOR, TAKE_ELEVATOR_OCCUPIED, SEARCH_ELEVATOR_PATH_FAILED,

ENTER_ELEVATOR_PATH_FOUND ]

DeliveryEventType   string

START FROM DOCK 从桩上出发执⾏任务

DELIVERY SETTINGS CHANGED  配送相关的设置项有更新

DELIVERY TASK START  开始执⾏配送任务

STATION TASK START  开始执⾏货柜取物任务

OPERATING CABINET 正在操作货柜

BACK TO RECEPTION FOR_FAILED_ORDER  配送失败回到前台

DELIVERY NO PICKUP 配送任务⽤户未取物

COLLECT NO PICKUP 回前台任务⽤户未取物

.    UNDOCK FAILED  下桩失败(下桩重试时间为2分钟)

·     START TO WORK  机器⼈开始⼯作(进⼊⼯作时间或电量充到80%)

GET OFF WORK 机器⼈下班啦

LOW BATTERY  电量过低，机器⼈即将回桩

NEW TASK RECEIVED  从云端接收到新的任务

ROBOT REBOOT 机器⼈即将重启

DISINFECT TASK FAILED 消毒任务失败

LOCALIZATION_ANOMALY 定位异常 , 需要推回桩或调⽤接⼝重置机器⼈位姿后才会消除

Enum:

[ START_FROM_DOCK, DELIVERY_SETTINGS_CHANGED, DELIVERY_TASK_START, STATION_TASK_START,

OPERATING_CABINET, BACK_TO_RECEPTION_FOR_FAILED_ORDER, DELIVERY_NO_PICKUP, COLLECT_NO_PICKUP, UNDOCK_FAILED, START_TO_WORK, GET_OFF_WORK, LOW_BATTERY, NEW_TASK_RECEIVED, ROBOT_REBOOT,

DISINFECT_TASK_FAILED, LOCALIZATION_ANOMALY ]

RobotEvent

{

description:

机器⼈事件信息，type在不同场景下会扩展新的定义，APP只需处理⾃⼰关⼼的事件即可。 GeneralEventType为通⽤的事件，ElevatorEventType为进出电梯相关的事件 ,

DeliveryEventType为配送相关事件。

type

timestamp

{...}

string($int64)

系统启动以来的毫秒数

}

PostTaskRequestEntry      {

location

type

{...}

string

TAKEOUT 外卖配送任务（仅限有货仓的机型）

GUIDE 引领任务   将⼈带到指定⽬的地

FOOD DELIVERY 送餐任务

RETURN 快速返航，回到取餐点

RECYCLE 回收餐盘

TAKEOUT_DISTRIBUTE 外卖分发，打开所有舱⻔由⽤户⾃主取物

Enum:

Array [ 6 ]

cargos }

[...]

DeliveryWorkMode      {

work_mode            string

DISPATCH 派送模式，本地应当禁⽌⽤户创建任务，只响应云端的呼叫任务

RECYCLE 回盘模式，本地除了回盘禁⽌创建其他任务，可响应云端的呼叫回盘任务   MANUAL ⼿动操作模式，本地⼈⼯创单进⾏配送或回盘

Enum:

Array [ 3 ]

}

{...}

{...}

DeliverySettings      { low_battery_level  timeout_settings

}

{

WorkingTime

[...]

[...]

hours

restdays

}

LocationInfo

poi_name type

{

string

string

PoiType

Enum:

Array [ 7 ]

}

GoingHomeStage

description:

stage

milestone

current_floor

target_floor

}

IdleStage      {

description:

stage

current_floor

}

{

正在回桩，对应stage为GOING_HOME

string

Enum:

Array [ 1 ]

string

Enum:

Array [ 8 ]

string

example: 2F

string

example: 1F

空闲，对应stage为IDLE

string

Enum:

Array [ 1 ]

string

example: 1F

OnDeliveringStage

description:

{

正在前往⽬标点，对应stage为ON_DELIVERING

stage

milestone

current_floor

target_floor

info

}

ArrivedAtTargetStage

description:

stage

info

pickup

}

DeviceErrorStage      { description:

stage

info }

string

Enum:

Array [ 1 ]

DeliveryTaskEventStage string

任务执⾏阶段

Enum:

Array [ 13 ]

string

string

{...}

{

已到达⽬标点，对应stage为ARRIVED_AT_TARGET

string

Enum:

Array [ 1 ]

{...}

DeliveryTask

{...}

设备故障，对应stage为DEVICE_ERROR

string

Enum:

Array [ 1 ]

{...}

OnReturningStage

description:

stage

current_floor

target_floor

{

机器⼈正在返航

string

Enum:

Array [ 1 ]

string

string

}

ArrivedAtTaskPointStage      { description:

到达任务点，对应stage为ARRIVED_AT_TASK_POINT

stage

string

Enum:

Array [ 1 ]

info }

{...}

GoingToTaskPointStage      { description:

正在前往任务点，对应stage为GOING_TO_TASK_POINT

stage

string

Enum:

Array [ 1 ]

current_floor

target_floor

info

string

string

{...}

}

TaskStage      {

description:

oneOf ->

返回数据为以下⼏种结构体中的其中—种，⾸先判断stage字段，再解析剩余的信息 DeviceErrorStage      {...}

{...}

{...}

GoingToTaskPointStage  ArrivedAtTaskPointStage

OnDeliveringStage      {...}

ArrivedAtTargetStage      {...} OnReturningStage      {...}

GoingHomeStage      {...} IdleStage      {...}

}

ElevatorInfo

{

description:

电梯区域信息

door_type*

elevator_id*

front_scheduling_poses

rear_scheduling_poses

}

string

Enum:

Array [ 3 ]

string

[...]

[...]

MoveOptions      { mode*

integer

default: 0

0： ⾃由导航，  1：严格轨道模式(遇障碍物停⽌并等待)，2：  轨道优先模式(遇障碍物下轨绕⾏)

Enum:

Array [ 3 ]

flags

yaw

[...]

number

到⽬标点后机器⼈的朝向

acceptable_precision

number

可接受的到点范围，当⽬标点被占据时，机器⼈离⽬标点距离在该范围内都算成功， 默认值为0.1⽶ 或0.18⽶ ，该值不影响机器⼈到点精度。

fail_retry_count

integer

失败重试次数

speed_ratio

number

【所需固件版本  4.5.4】速度⽐例 , 配置的最⼤移动速度乘以该值为本次运动的最⼤速度 , 最⼩值为

0.1, ⼤于1的值会导致避障距离变⻓ ，在动态障碍物较多的环境中请谨慎使⽤。

}

EnterElevatorOptions      {

elevator_door_flag   string

default: front_door

items: OrderedMap { "type": "string", "enum": List [ "front_door",

"rear_door" ] }

front door 从电梯前⻔进⼊

rear_door 从电梯后⻔进⼊

elevator_stopping_yaw string

default: face_to_front_door

items: OrderedMap { "type": "string", "enum": List [ "face_to_front_door",

"face_to_rear_door" ] }

face to front door  进⼊电梯后⾯向电梯前⻔

face_to_rear_door 进⼊电梯后⾯向电梯后⻔

timeout_in_ms        number

进电梯的总时⻓

use_conservative_mode boolean

true表示保守策略，前往电梯中⼼点。false表示挤电梯，前往电梯更⾥⾯

}

LeaveElevatorOptions      { elevator_door_flag

string

default: front_door

items: OrderedMap { "type": "string", "enum": List [ "front_door", "rear_door" ] }

front door 从电梯前⻔进⼊

rear_door 从电梯后⻔进⼊

timeout_in_ms

number

出电梯的总时⻓

arrive_door_timeout_in_ms

number

出电梯过程中到达电梯⻔的超时时间

search_path_timeout_in_ms

number

出电梯的搜路超时时间，超过这个时间仍未搜到路便放弃出电梯

on_elevator_door_timeout_in_ms number

if_need_reach_milestone

move_options

}

堵在电梯⻔槛上的超时时间，到达超时放弃出梯

boolean

true前往出后前往⽬标点，false表示只到达⻔⼝等待点

MoveOptions      {...}

MoveToActionOptions      {

target*              Location      {...}

move_options         MoveOptions      {...}

}

SeriesMoveToActionOptions      {

[...]

MoveOptions

targets*

{...}

move_options }

integer

ActionDirection

0 前进

1 后退

2 右转

3 左转

Enum:

[ 0, 1, 2, 3 ]

MoveByActionOptions

description:

{

遥控运动的参数，direction和theta只需包含—个即可，前者指定⽅向，后者指定—个角速度进⾏ 转弯

direction            ActionDirection integer

0 前进

1 后退

2 右转

3 左转

Enum:

Array [ 4 ]

theta                number

duration             integer

运动持续时间，单位毫秒。不指定时默认持续500毫秒。注意！遥控运动不会避障，请勿设置过⻓的持 续时间。

}

{

{...}

GoHomeActionOptions

gohome_options

}

RotateActionOptions      {

angle*               number

机器⼈转动的⻆度值，单位弧度，正数表示逆时针旋转，负数表示顺时针旋转

}

RotateToActionOptions      {

angle*               number

机器⼈停⽌时的Yaw值，单位弧度

}

RecoverLocalizationActionOptions      { description:

不包含area字段或area为空时为全局重定位，否则为局部重定位

area

Rectangle      {...}

relocalization_options

{...}

}

MoveToTagActionOptions      { description:

标签精准对接，其中target为开始对接的起始位置

target*

tag_type*

target_relative_pose backward_docking

turn_radian

tag_ids

reflect_tag_num

dock_retry_count

dock_allowance

Pose3D      {...}

integer

default: 0

0：⼆维码视觉标签，  1：激光标签 , 2：激光反光板 , 3：货架(需要6.0或更⾼版本)

Enum:

Array [ 4 ]

{...}

boolean

default: false

是否向后对接

number

对接成功后的转向弧度，默认机器是⾯向或背对Tag，如果需要机器在对接成功后转指定⻆度，请设置 该字段。

[...]

integer

tag_type为2时有效，对接的反光板个数，默认为1.

integer

对接失败后的重试次数，默认不重试 .

number

tag_type为3时有效，对接货架时默认机器中⼼对准货架中⼼ 。dock_allowance表示机器⼈留在货 架外的机身⻓度。

}

BackOfFromTagActionOptions      { description:

从⼆维码前后退

backup_mode          integer

0  ⾃由后退

1 窄道后退，在后退过程中⼀直观测⼆维码并调整⻆度

Enum:

Array [ 2 ]

tag_type             integer

default: 0

0：⼆维码视觉标签，  1：激光标签 , 2：  激光反光板

Enum:

Array [ 3 ]

backup_distance      number($double)

后退的距离 , 可选值，默认后退直到机器可以转身

backward_docking     boolean

default: false

是否向后对接 , 如果是向后对接，则调⽤BackOffFromTagAction时实际是向前移动

}

MultiFloorMoveActionOptions      {

{...}

MoveOptions

target*

{...}

move_options

}

{

清扫Action的参数，不包含region_ids时清扫所有区域，否则只清扫指定区域

{...}

SweepActionOptions

description:

sweep_options

}

ReturnToParkingActionOptions      { description:

返航Action的参数

target

move_options }

{...}

{...}

MoveOptions

FollowPathPointsActionOptions      {

[...]

MoveOptions

path_points*

{...}

move_options }

EnterElevatorActionOptions      {

elevator_id*          string

{...}

EnterElevatorOptions

enter_elevator_options

}

LeaveElevatorActionOptions      { elevator_id*          string

target                Pose2D      {...}

{...}

LeaveElevatorOptions

leave_elevator_options

}

DisabledSensorMaskData      {

id                   integer

isAlways             boolean

}

SensorMaskCtrlData

id

isAlways

isEnabled

}

{

number

boolean

boolean

LightControlData      { channel

controlPart

mode

color

brightnessEndColor

brightMs

offMs

}

{

TaskOperation

display_name

action

wait_time

}

TaskTemplateRequest

key

name

action_list }

string

example: One

led控制通道:One通道⼀ ，Two通道⼆

Enum:

Array [ 2 ]

string

example: Left

led控制部分:Left左半部，Right右半部 Enum:

Array [ 2 ]

string

example: AlwaysBright

led控制模式:AlwaysBright常亮，Breathe呼吸，Blink闪烁，HorseLamp跑⻢ Enum:

Array [ 4 ]

{...}

{...}

integer

常亮模式可填⼊任意值；呼吸模式填⼊亮度单次变化时间（ 单次变化表示color每次增⼤1的时间）；闪 烁模式填⼊点亮的持续时间；跑⻢模式表示点亮下⼀个灯的时间

integer

闪烁模式填⼊熄灭的持续时间；其他模式可填⼊任意值

string

⽬标点位POI的display_name

string

机器⼈在⽬标点位需要执⾏的操作

integer

机器⼈在⽬标点位完成操作后的等待时间，单位秒

{

string($uuid)

任务模板ID

string

任务模板名称

[...]

TaskTemplate      {  task_template_key

task_template_type

name

scene_id

device_id

action_list }

{

IndustryTarget

target_name

action

wait_time

}

IndustryTaskRequest

target

type

order_id

template_key start_time

task_targets

message

string($uuid)

任务模板ID

integer

任务模板类型

string

任务模板名称

string($uuid)

任务模板绑定的场景ID

string($uuid)

创建任务模板的设备ID

[...]

string

string

integer

{

string

string

Enum:

Array [ 3 ]

string

string

string

[...]

{...}

}

string

IndustryTaskEventStage

任务执⾏阶段

Enum:

Array [ 10 ]

{

IndustryTaskResult

string

IndustryTaskEventStage

stage

任务执⾏阶段

Enum:

Array [ 10 ]

reason

timestamp

string

default:

string($datetime)

}

| IndustryTask id task status result } | { string($uuid) IndustryTaskRequest {...} DeliveryTaskStatus string Enum: Array [ 6 ] IndustryTaskResult {...} |
| --- | --- |